THEATRICAL DCP CHECKLIST

July 2022

Supplemental to International Broadcast Deliverables List

For questions please contact: Luke.Harman@fremantle.com

All material must be technically assessed by the production company prior to delivery which is a contractual obligation. Any material found to be non-compliant with our technical delivery standards will be rejected at the point of delivery

Deliveries must be made as electronic transfers to a Signiant Media Shuttle portal designated by Fremantle

PROMOTIONAL AND MARKETING SUPPORT ASSETS

- Press-book (in Italian and in English)

- Synopsis and above-the-line crew

- Final and approved cast List

- Director’s biography (in Italian and English)

- Definitive below-the-line crew

- High resolution set photographs

- High resolution photograph of the director

- Billing Block for advertising material

- Final Italian film dialogue list (spotting list with timecodes)

DOCUMENTS FOR ITALIAN THEATRICAL RELEASE

- List of opening and closing credits

- © Copyright (Production Year and Production Formula)

- Italian Censorship visa

- Certificate of Expression of Italian Origin (EOI)

DELIVERY OF MATERIALS FOR ITALIAN THEATRICAL RELEASE

DCP FEATURE 24fps (INTEROP)

If this is native 25fps content;

DCP FEATURE 25fps (STANDARD SMPTE) and also deliver DCP 24fps (INTEROP)

- FEATURE H.264 STEREO + SUBTITLES (.mov wrapper, H.264 codec at 15mbps, stereo audio mix, no subtitles burnt into video, synchronised .STL and .SRT files) this file must conform exactly to the DCP FEATURE

- Distribution KDM (DKDM) VALID FOR 5 YEARS (expiration on 31st December of the 5th year)

- DVD and BLURAY ISOs conforming to the distribution DCP

TECHNICAL SPECIFICATIONS FOR ITALIAN THEATRICAL RELEASE

- CPL, PKL , ANNOTATION TEXT must comply with the DIGITAL NAMING CONVENTION

- Required audio channels are: L-R-C-LFE-LS-RS

- The opening sequence of the film must be divided into separate reels. One Logo = one Reel. First to last opening credit = one Reel. If the opening credits are over scenes, always also provide the textless TIFF XYZ

- If forced subtitles are present in the film, these must not be burnt into the image and must be delivered separately in .xml INTEROP or SMPTE subtitle formats

DELIVERY OF MATERIALS FOR HOME ENTERTAINMENT

- H.264 of the film with visual TIMECODES in reels (same as DCP)

- ProRes FEATURE original film speed (see ProRes delivery specifications)

- H.264 single-reel film with no visual TIMECODES and no watermark, conforming to ProRes

- TIFF XYZ Film and ProRes Backgrounds

- 5.1 printmaster, 5.1 M&E. 5.1 DME stems at 24 fps DCP with reels conforming to the DCP and single reel. Printmaster and M&E also at 25 fps

- STL/SRT/DOC file of Italian subtitles for the deaf synchronised with ProRes

- Audio description in Italian synchronised with ProRes

- STL/SRT/DOC FILE of forced subtitles if applicable

- Backstage, deleted scenes, cast & crew interviews - if available - in ProRes 422HQ original aspect ratio and original speed

- Director's commentary on the film - if available and in all formats produced

TECHNICAL SPECIFICATIONS FOR HOME ENTERTAINMENT

PRORES FEATURES

- PRORES 422 HQ 1920X1080 (if available, also PRORES 422HQ UHD SDR&HDR) original film speed (the logo and opening credits sequence is different from DCP and must be sent via Vimeo link to VISION for approval)

- Any forced subtitles must not be imprinted but delivered separately as synchronised STL and SRT files

- 8-CHANNEL AUDIO (L R C LFE LS RS LT RT)

- TC START 59:59:00 - 1 SECOND OF BLACK SCREEN

- 01:00:00 PROGRAMME - 1 SECOND OF BLACK SCREEN AT THE END

- TEXTLESS PRORES to be delivered separately

MATERIAL DELIVERY FOR INTERNATIONAL DISTRIBUTION

∙ TIFF XYZ TEXTLESS (already present in normal deliveries)
∙ MIX 5.1, M&E 5.1 and DME STEMS in compliance with the DCP (already present in normal deliveries)
∙ Feature MOV H.264 in compliance with the DCP visual timecode burn in TC (please keep the division in reels of the DCP) no burnt-in subtitles
∙ File STL/SRT/DOC/XML INTEROP-SMPTE of the Italian subtitles for hearing impaired synchronized with the ProRes

∙ File STL/SRT/DOC/XML INTEROP-SMPTE of english subtitles
∙ File STL/SRT/DOC/XML INTEROP-SMPTE of every other language (if available)
∙ If applicable FILE STL/SRT/DOC of Italian forced subtitles
∙ WAV of the soundtrack free of rights

TECHNICAL SPECIFICATIONS FOR FOREIGN THEATRICAL SALES

DCP FEATURE

- The opening logos sequence of the film must be divided into a separate reel according to the specific instructions that will be provided:

- If the film features forced Italian subtitles, they should not be burnt into the image

FEATURE PRORES HD and UHD

- PRORES 1920X1080 4444 original film speed / Any separate forced Italian subtitles must be in STL and SRT format

- PRORES 4444 UHD original film speed / Any separate forced Italian subtitles must be in STL and SRT format

- AUDIO 16 CHANNELS:

1) L

2) R

3) C

4) LFE

5) LS

6) RS

7) LT

8) RT

9) M&E 5.1 L

10) M&E 5.1 R

11) M&E 5.1 C

12) M&E 5.1 LFE

13) M&E 5.1 LS

14) M&E 5.1 RS

15) M&E 2.0 L

16) M&E 2.0 R

- TC START 59:59:00 – 1 SECOND OF BLACK SCREEN

- 01:00:00:00 PROGRAMME – 1 SECOND OF BLACK SCREEN AT THE END

- PRORES of TEXTLESS elements

- separate DME synchronised with the PRORES